Setup of the Grandstream GXP1625 And How to Use

Section 1: Physical Setup

- Begin by removing everything from the box. Then, attach the stand to the back of the main base.

- Next, connect the phone cord (swirled string) to the main base and then to the handset of the phone. Place the handset onto the receiver spot.

- Next, connect the Ethernet cable to either the POE port or the regular Ethernet port. Finally, connect the power block and cord to the circular spot in the middle.

- Plug your Ethernet cord into the internet block that you are using and the power block into an outlet. It should now power on.

Finished Setup:

Section 2: How to Use Phones

- Phone Overview

Key Parts

- LCD Screen – Displays call info, time, and menus

- Soft Keys – Buttons under the screen that change function

- Line Keys (2) – Used for active lines/accounts

- Navigation Keys – Up/down/left/right + OK

- Dial Pad – Standard number keypad

- Speaker Button – Enables speakerphone

- Headset Button – Activates headset mode

- Mute Button – Mutes microphone

- Volume Keys – Adjust ringer/call volume

2. Making Calls

Method 1: Handset

- Pick up the handset

- Dial the number

- Wait for the call to connect

Method 2: Speakerphone

- Press the Speaker button

- Dial the number

- Speak hands-free

Method 3: Pre-Dial

- Enter the number while on-hook

- Press Dial or lift the handset

3. Answering Calls

- Handset: Pick up the handset

- Speaker: Press the Speaker button

- Headset: Press the Headset button

4. Ending Calls

- Hang up the handset

- OR press End Call soft key

- OR press the Speaker/Headset button

5. Call Features

Hold

- Press Hold during a call

- Press Resume to return

Mute

- Press Mute to silence your microphone

- Press again to unmute

Transfer Call

Blind Transfer (immediate)

- Press Transfer

- Dial the number

- Press Transfer again

Attended Transfer (announce first)

- Press Transfer

- Dial the number

- Wait for answer

- Press Transfer again

Conference Call (3-way)

- During a call, press Conference

- Dial the second number

- Press Conference again to join all parties

6. Call History

View Call Logs

- Press Menu

- Select Call History

- Choose:

  - Missed Calls

  - Received Calls

  - Dialed Calls

Redial

- Press the Redial button to call the last number

7. Contacts (Phonebook)

Access Contacts

- Press Menu

- Select Phonebook

Add Contact

- Go to Phonebook

- Select Add Contact

- Enter name and number

- Save

Call a Contact

- Open Phonebook

- Select contact

- Press Dial

8. Volume Control

- During a call: adjusts call volume

- When idle: adjusts ringer volume

9. Ringtones

- Press Menu

- Go to Preferences → Ring Tone

- Select a tone

- Save

10. Do Not Disturb (DND)

- Press the DND soft key to enable

- Press again to disable

When enabled, incoming calls will not ring

11. Troubleshooting

No dial tone

- Check network connection

- Ensure phone is registered

Can’t hear audio

- Check volume

- Ensure correct device (handset/speaker/headset)

Calls not going through

- Verify account/SIP registration with admin

12. Quick Tips

- Use Redial for speed

- Use Hold + Transfer for call handling

- Keep volume adjusted to avoid missed calls

- Learn soft key labels—they change depending on context